CSE 4571 – PRACTICAL ROBOTICS PROJECTS WITH ARDUINO

EXPERIMENT 1
Ultrasonic Sensor-Based Distance Measurement and Obstacle Detection Using Arduino

| Duration | Level | Mode | CO Mapping |
| --- | --- | --- | --- |
| 2 Hours | B.Tech CSE Final Year | Hardware / Wokwi | CO1, CO2, CO6 |

1. Aim of the Experiment

To interface an HC-SR04 ultrasonic sensor with an Arduino UNO, measure object distance, and implement threshold-based obstacle detection using LEDs and a buzzer.

2. Learning Objectives

Understand ultrasonic time-of-flight distance measurement.

Interface a digital distance sensor with Arduino UNO.

Acquire echo duration and convert it into distance.

Implement threshold-based decision making.

Use LED and buzzer outputs as visual and audible alerts.

Develop confidence in basic sensor–controller–actuator integration.

3. Components (TinkerCAD/ Wokwi & Hardware Parts)

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Microcontroller |
| HC-SR04 Ultrasonic Sensor | 1 | Distance measurement |
| Red LED | 1 | Danger indication |
| Yellow LED | 1 | Warning indication |
| Active Buzzer | 1 | Audible alert |
| 220 Ω Resistor | 2 | LED current limiting |
| Breadboard | 1 | Circuit assembly |
| Jumper Wires | As required | Interconnections |

4. Theory

The HC-SR04 emits an ultrasonic burst at approximately 40 kHz through its TRIG interface. The sound wave reflects from an object and the ECHO pin remains HIGH for a duration proportional to the round-trip travel time. Arduino measures this pulse width and converts it to distance.

Distance (cm) = Echo time (µs) / 58

For this experiment, three simple decision zones are used: 0–10 cm = DANGER, 10–25 cm = WARNING, and above 25 cm = SAFE.

5. Pre-Lab Questions

What is the operating principle of an HC-SR04 sensor?

Why are separate TRIG and ECHO pins required?

What is the approximate speed of sound in air?

Why is pulseIn() used in this experiment?

Why should LEDs be connected through current-limiting resistors?

6. Circuit Diagram Implemented Using TinkerCAD/Wokwi

Figure 1. Software Implemented Ultrasonic Sensor-Based Distance Measurement and Obstacle Detection.

7. Circuit Diagram Implemented Using Hardware’s

Figure 2. Hardware Implemented Ultrasonic Sensor-Based Distance Measurement and Obstacle Detection.

8. Connection Table

| Device Pin | Arduino Pin | Connection / Note |
| --- | --- | --- |
| HC-SR04 VCC | 5V | 5 V supply |
| HC-SR04 TRIG | D9 | Digital output |
| HC-SR04 ECHO | D10 | Digital input |
| HC-SR04 GND | GND | Common ground |
| Red LED (+) | D6 | Through 220 Ω resistor |
| Yellow LED (+) | D5 | Through 220 Ω resistor |
| Buzzer (+) | D7 | Buzzer (−) to GND |

9. Procedure

1. Place the Arduino UNO and components on the breadboard.

2. Complete all power and ground connections first.

3. Connect the HC-SR04 TRIG and ECHO pins to D9 and D10 respectively.

4. Connect the LEDs through 220 Ω resistors and connect the buzzer to D7.

5. Open Arduino IDE or the corresponding Wokwi simulation.

6. Enter and compile the program given below.

7. Upload/run the program and open Serial Monitor at 9600 baud.

8. Vary the target distance and record at least five observations.

9. Verify that the LED and buzzer responses agree with the defined distance zones.

10. Arduino Program

11. Observation Table

| Sl. No. | Actual Distance (cm) | Measured Distance (cm) | Error (cm) | Zone | Buzzer State |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

12. Result

The HC-SR04 ultrasonic sensor was successfully interfaced with Arduino UNO. Object distance was measured and classified into safe, warning, and danger zones, with corresponding LED and buzzer indications.

13. Post-Lab / Application Questions

Calculate the percentage error for any three readings.

Modify the thresholds to 15 cm and 30 cm and verify the outputs.

How would you reduce false triggering caused by noisy distance readings?

How can this circuit be extended into an autonomous obstacle-avoiding robot?

What changes would be required to control a DC motor based on the measured distance?

14. Student Record

| Student Name | Registration No. | Date | Faculty Signature |
| --- | --- | --- | --- |