CSE 4571 – PRACTICAL ROBOTICS PROJECTS WITH ARDUINO

EXPERIMENT 2
ANALOG SENSOR INTERFACING AND REAL-TIME DATA ACQUISITION

| Duration | Level | Mode | CO Mapping |
| --- | --- | --- | --- |
| 2 Hours | B.Tech CSE Final Year | Hardware / Wokwi | CO1, CO2, CO5 |

1. Aim of the Experiment

To interface an analog sensor with Arduino UNO, acquire ADC data and study its response to changing illumination.

2. Learning Objectives

Understand ADC-based acquisition.

Read a Wokwi photoresistor.

Observe sensor response at different lux values.

Use Serial Monitor/Serial Plotter for visualization.

3. Components / TinkerCAD or Wokwi Parts

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Microcontroller / ADC |
| Photoresistor module | 1 | Analog illumination sensor |
| LED | 1 | Threshold indication |
| 220 Ω resistor | 1 | LED current limiting |
| Breadboard | 1 | Circuit assembly |
| Jumper wires | As required | Interconnections |

4. Theory

Arduino UNO provides 10-bit analog inputs. The Wokwi photoresistor module exposes AO, whose voltage varies with simulated illumination. This experiment introduces acquisition, thresholding and simple calibration.

ADC value = analogRead(A0)  →  0 to 1023

Approx. voltage = ADC × 5.0 / 1023.0

5. Pre-Lab Questions

What is ADC resolution?

What does analogRead() return?

Why is calibration important?

Why is A0 used for the photoresistor signal?

5. Circuit Diagram

Figure 1. Arduino UNO, TinkerCAD/Wokwi photoresistor module and LED threshold indicator.

6. Connection Table

| Component / Signal | Arduino Pin | Connection Details |
| --- | --- | --- |
| Photoresistor AO | A0 | Analog input |
| Photoresistor VCC | 5V | Supply |
| Photoresistor GND | GND | Common ground |
| LED Anode (+) | D2 | Through 220 Ω resistor |
| LED Cathode (−) | GND | Ground |

7. Circuit Diagram Implemented Using Hardware’s

8. Procedure

- Add the photoresistor and LED to the Arduino UNO Wokwi project.

- Connect AO pin of photoresistor module to A0 of Arduino Uno and the LED to D2 through a 220 Ω resistor.

- Complete 5V and GND connections and verify common ground.

- Read A0 and calculate the approximate voltage.

- Change the simulated illumination/lux value while the simulation is running.

- Record ADC values and observe the trend in Serial Monitor/Serial Plotter.

- Take at least five readings and record the LED state.

- Plot or inspect ADC against simulated illumination.

9. Arduino Program

10. Observation Table

| Sl. No. | Simulated Lux | ADC Value | Voltage (V) | LED State |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

11. Expected Output / Analysis

The Serial Monitor displays the ADC value and the corresponding calculated voltage. The LED is switched ON when the ADC reading is below 500 and OFF otherwise.

| ADC Condition | LED | Interpretation |
| --- | --- | --- |
| ADC < 500 | ON | Below programmed threshold |
| ADC ≥ 500 | OFF | At/above programmed threshold |

12. Result

The analog photoresistor was successfully interfaced with Arduino UNO. ADC data and corresponding sensor voltage were acquired, and the LED threshold response was verified in Wokwi.

13. Post-Lab / Application Questions

Plot ADC against simulated illumination and describe the trend.

Change the threshold and compare the results.

Why can sensor response be nonlinear?

How could this sensor be used for automatic lighting in a robot or smart environment?

14. Precautions

Verify pin assignments before running the simulation.

Use common ground for connected devices.

Record both the input condition and observed output.

For physical implementation, consider current, voltage, mechanical loading and sensor tolerances.

15. Suggested Extension

Modify the program to add one meaningful feature, document the change, and preserve the original behaviour as a baseline.

16. Student Record

| Student Name | Registration No. | Date | Faculty Signature |
| --- | --- | --- | --- |