CSE 4571 – PRACTICAL ROBOTICS PROJECTS WITH ARDUINO

EXPERIMENT 3
SERVO MOTOR CONTROL FOR POSITION-BASED ROBOTIC ACTUATION

| Duration | Level | Mode | CO Mapping |
| --- | --- | --- | --- |
| 2 Hours | B.Tech CSE Final Year | Hardware / Wokwi | CO2, CO3 |

1. Aim of the Experiment

To control a micro-servo using a potentiometer as a position command.

2. Objectives

Interface a servo.

Read an analog command.

Map ADC values to angular position.

Demonstrate a robotic actuator.

3. Components / TinkerCAD/ Wokwi Parts

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Microcontroller |
| Micro Servo | 1 | Position-controlled actuator |
| Potentiometer | 1 | Analog position command |
| Breadboard | 1 | Circuit assembly |
| Jumper wires | As required | Interconnections |

4. Theory

A hobby servo interprets control pulses as a target position. The Arduino Servo library provides a simple interface. The potentiometer provides an analog command at A0, and its ADC reading is mapped from 0–1023 to a servo command of 0–180 degrees.

ADC range: 0–1023  →  Servo command range: 0–180°

5.Pre-Lab Questions

What is PWM?

Why is a servo different from a DC motor?

What does map() do?

5. Circuit Diagram

Figure 1. Verified potentiometer-to-servo position-control wiring.

6. Connection Table

| Component / Signal | Arduino Pin | Connection Details |
| --- | --- | --- |
| Servo signal | D9 | Position-control signal |
| Potentiometer SIG | A0 | Analog command |
| Potentiometer VCC | 5V | Supply |
| Potentiometer GND | GND | Ground |
| Servo VCC | 5V | Supply |
| Servo GND | GND | Ground |

7. Circuit Diagram Implemented Using Hardware’s

8. Procedure

- Place Arduino UNO, potentiometer and micro servo in the Wokwi project.

- Wire the circuit exactly as shown in Figure 1.

- Upload/run the program.

- Open Serial Monitor at 9600 baud.

- Move the potentiometer through its range.

- Observe the servo movement and corresponding commanded angle.

- Record five ADC/expected-angle/observed-angle pairs.

- Restrict the operating range as an exercise.

9. Arduino Program

10. Observation Table

| Sl. No. | ADC | Expected Angle (°) | Observed Angle (°) |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

11. Expected Output / Analysis

As the potentiometer is rotated, the ADC reading changes. The program maps this input to approximately 0°–180°, and the servo moves toward the corresponding position. The Serial Monitor reports the ADC reading and commanded angle.

12. Result

The micro-servo was successfully controlled using the potentiometer as an analog position command. The ADC-to-angle mapping and corresponding servo movement were verified in Wokwi.

13. Post-Lab Questions

Restrict the servo angle to 20–160 degrees.

Add preset positions using a pushbutton.

What does Servo.attach() do?

Why is position control useful in robotics?

14. Precautions

Verify pin assignments before running the simulation.

Use common ground for connected devices.

Do not short the potentiometer signal node to 5V or GND.

Record both the input condition and observed output.

For physical implementation, verify servo supply capability, current demand and mechanical loading.

15. Suggested Extension

Modify the program to add one meaningful feature, document the change, and preserve the original behavior as a baseline.

16. Student Record

| Student Name | Registration No. | Date | Faculty Signature |
| --- | --- | --- | --- |